Trail sign notes:

Make template using outline font; saves printer ink

Font - Helvetica 90 point

Spacing - single

Arrows - use 1/4 router bit

Letters - using 3/16 router bit

Routing depth ~ 1/8”

Use 1X6 PT wood; mostly 16” long

Attach the template to the wood using packing tape along all 4 sides.

Dark letters - Rust-oleum Ultra Cover Premium Latex paint, Kona Brown

White background - Cabot acrylic stain, Cape Code Grey.

Mount: 9” long X 2” Azec or Trex scraps.

DEAD END PRIVATE LAND

AHEAD

BARRETTS HILL

SUMMIT

460 FEET

Sign experiments

BARRETTS

Sign experiments

HILL

Haynes-

Wheeler Gift

v

BOLTON
CONSERVATION COMMISSION

Sign experiments

DANFORTH

Sign experiments

LANE

DANFORT

DANFORTH

LANE

CIRCLE

DANFORTH

Sign experiments

LANE

PARKING